Norwegian Unsiversity of Science and Technology

Application Programming Interface

Njord

Espen Tolpinrud

07.06.2012

Revision: 1.0.0

This document is meant to be used as an Application Programming Interface (API) for future control system developers.

Table of Content

Introduction	3

Objects	4

Vessel.lvclass	4

Supervisor.lvclass	9

Thrust Allocation.lvclass	21

Thruster.lvclass	22

Sensor.lvclass	23

Observer.lvclass	24

Kalman Filter.lvclass	25

Kalman Filter Altitude.lvclass	26

Passive Observer.lvclass	27

Adaptive Passive Observer.lvclass	29

Guidance.lvclass	30

Altitude Guidance.lvclass	31

DP Guidance.lvclass	32

Joystick Guidance.lvclass	33

Tracking Guidance.lvclass	36

Tracking STRM Guidance.lvclass	37

Controller.lvclass	39

Joystick Controller.lvclass	40

PID Controller.lvclass	41

Linear PID Controller.lvclass	42

Non Linear PID Controller.lvclass	43

LQR Controller.lvclass	44

Sliding Mode.lvclass	45

Backstepping.lvclass	46

Adaptive Backstepping.lvclass	47

Stand-Alone Methods	48

mainVI.vi	48

mainVI_SIL.vi	48

Reference Model	48

Kinematics	48

Integrators	50

Signal Processing	50

Others	51

Configuration File	53

Table of tags and attributes	53

Introduction

It is recommended to have some basic knowledge of LabView and programming before reading this documentation.

Explanation of expressions

Cluster is a gathering of value types much like struct in other programming languages.

Matrix is a typedef 2D Array of Double
Complex Matrix is a typedef of 2D Array of Complex Numbers.

Objects

Vessel.lvclass

Labview Object.lvclass
	∟ Vessel.lvclass

Implemented by Espen Tolpinrud.

The Vessel class is a representation of the craft in the control system.

Constructor summary

| Name | Note |
| --- | --- |
| Vessel() | Creates a Vessel Object where the TCP port number is set to default, 8500. |
| Vessel(UInt 16 port) | Creates a Vessel Object where the TCP port number is set to be the given port |
| Vessel(FPGA VI Ref) | Creates a Vessel Object where the FPGA VI Ref is set. The TCP ports is set to default |

Field summary

| Data Type | Name | Note |
| --- | --- | --- |
| 1D Array of Observer.lvclass | Array Altitude Observer | Array of Observer objects for Altitude |
| 1D Array of Controller.lvclass | Array Controllers | Array of Controller objects |
| 1D Array of Observer.lvclass | Array Observer | Array of Observer objects for position, velocity and acceleration estimation. |
| Double | Buoyancy | The Buoyancy value for the vessel |
| 1D Array of Double | CB | The Center of Buoyancy for the vessel (Maybe to be removed) |
| 1D Array of Double | CG | The Center of Gravity for the vessel (Maybe to be removed) |
| Cluster of {UInt16,1D Array of Double} | Coordinate Reference | Contains the type of coordinate system to be used together with an array of offset values. |
| Matrix | Coriolis_RB | A Matrix with Coriolis effects for the vessel |
| Int32 | DOF | The number of Degrees of Freedom in the system |
| Int32 | DP Altitude Observer Index | The Index pointing to the observer object to be used from Array Altitude Observer during DP operation |
| Int32 | DP Controller Index | The Index pointing to the controller object to be used from the Array Controllers during DP operation |
| Int32 | DP Guidance Index | The Index pointing to the guidance object to be used from the Guidance Array during DP operation |
| Int32 | DP Observer Index | The Index pointing to the observer object to be used from the Array Observers |
| 1D Array of Int32 | Drop Out Array | Array indicating healthy and unhealthy sensors. |
| Cluster of {1D Array of Double, 1D Array of Int32} | DVL | Cluster containing information from the Doppler Velocity Log - Value (1D Array of Double) - Signal (1D Array of Int32) |
| FPGA Interface | FPGA VI Reference | A Reference to the FPGA VI implemented on the NI compactRIO. |
| Double | Height | The height of the vessel (Maybe to be removed) |
| Int32 | Joystick Guidance Index | The Index pointing to the guidance object to be used from the Guidance Array during Joystick Mode |
| Double | Length | The length of the vessel. (Maybe to be removed) |
| Matrix | Linear Damping | Matrix containing linear damping values for the vessel |
| Matrix | Mass_A | Matrix containing the Added Mass effects for the vessel |
| Matrix | Mass_RB | Matrix containing the Rigid Body Mass for the vessel |
| 1D Array of Double | Max Thrust | Array with maximum thrust in each degree of freedom for the vessel. |
| 1D Array of Double | Max_Acc | Array with maximum acceleration defined in body coordinates in each degree of freedom for the vessel. |
| String | Name | Name of the ROV |
| Matrix | NonLinear Damping | Matrix containing nonlinear damping values for the vessel. |
| 1D Array of Double | Nu_max | Array with maximum velocity defined in body coordinates in each degree of freedom for the vessel. |
| 1D Array of Cluster of {Int16, Int8, Int8, Int8, Int8} | Port Configuration | Array with communication information used to communicate with the NI compactRIO. The cluster contains the fields: - Baud Rate (Int16) - Data Bits (Int8) - Parity (Int8) - Stop Bits (Int8) - Flow Control (Int8) |
| Double | Sampling Time | The sampling time for the control loop. Determines the frequency of iteration. |
| Cluster of 4 {Cluster of {1D Array of Double, 1D Array of Double, 1D Array of Double, 1D Array of Double}, Cluster of { Double, Double, Double, Double, Double}, 1D Array of Double, 1D Array of Double, Cluster of {4 1D Array of Double}} | States | Contains the different states for the system - Measured (Cluster) - Eta (1D Array of Double) - Nu (1D Array of Double) - Eta_dot (1D Array of Double) - Nu_dot (1D Array of Double) - Estimated (Cluster) - Eta (1D Array of Double) - Nu (1D Array of Double) - Eta_dot (1D Array of Double) - Nu_dot (1D Array of Double) - Desired (Cluster) - Eta (1D Array of Double) - Nu (1D Array of Double) - Eta_dot (1D Array of Double) - Nu_dot (1D Array of Double) - Previous Desired (Cluster) - Eta (1D Array of Double) - Nu (1D Array of Double) - Eta_dot (1D Array of Double) - Nu_dot (1D Array of Double) - Tau (1D Array of Double) - Rpm (1D Array of Double) - Altitude (Cluster) - Measured - Estimated - Desired - Previous Desired - Approximated - Attitude - Theta Measured - Theta Estimated - Omega Measured - Omega Estimated |
| Supervisor.lvclass | Supervisor | A Supervisor object used to communicate with a user interface and ensure correct behavior in the system. |
| Thrust Allocation.lvclass | Thrust Allocation | A Thrust Allocation object used to keep track of the thrusters for the vessel |
| Int32 | Tracking Altitude Observer Index | The Index pointing to the observer object for altitude to be used from the Array Observer Altitude during Tracking |
| Int32 | Tracking Controller Index | The Index pointing to the controller object to be used from the Array Controllers during Tracking |
| Int32 | Tracking Guidance Index | The Index pointing to the guidance object to be used from the Guidance Array during Tracking |
| Int32 | Tracking Observer Index | The Index pointing to the observer object to be used from the Array Observers during Tracking |
| 1D Array of Sensor.lvclass | Used Sensors | Array containing all sensors used by the vessel. |
| Double | Width | The width of the vessel. (Maybe to be removed) |

Methods summary

| Data Type | Name with input arguments | Note |
| --- | --- | --- |
| Void | Add Element in Array Altitude Guidance{Guidance.lvclass new Altitude Guidance | Inserts a new altitude guidance object in the array. |
| Int32 | Add Element in Array Altitude Observer (Observer.lvclass new Altitude Observer) | Inserts a new Altitude Observer object in the Array Altitude Observer and returns the index of the new element. |
| Int32 | Add Element in Array Controller (Controller.lvclass new Controller) | Inserts a new Controller object in the Array Controllers and returns the index of the new element. |
| Int32 | Add Element in Array Guidance (Guidance.lvclass new Guidance) | Inserts a new Guidance object in the Array Guidance and returns the index of the new element. |
| Int32 | Add Element in Array Observers (Observer.lvclass new Observer) | Inserts a new Observer object in the Array Observers and returns the index of the new element. |
| Void | Add Element in Available Sensors (Sensor.lvclass new Sensor | Inserts a Sensor object in the Available Sensors array |
| Void | Add Element in Used Sensors (Sensor.lvclass new Sensor) | Inserts a Sensor object in the Used Sensors array. |
| Void | Control Structure () | Performs an iteration of the control loop. Must be placed in an outer loop structure in order to work properly. |
| Void | Discrete to Continuous (Void) | Transforms the angels in estimate and desired states from discrete to continuous. |
| Int32 | Find Controller by Name (String name) | Finds the index of the controller with the specified name in the controller array. If not present the method returns -1. |
| Int32 | Find Observer by Name (String name) | Finds the index of the observer with the specified name in the observer array. If not present the method returns -1. |
| Int32 | Find Sensor by ID (String Sensor ID) | Searches through the Used Sensors array and returns the Index of the desired Sensor, or -1 if no match is found. |
| Observer.lvclass | Get Active Altitude Observer () | Returns the Altitude Observer object for the active task. Default task is DP operation. |
| Controller.lvclass | Get Active Controller () | Returns the Controller object for the active task. Default task is DP operation. |
| Guidance.lvclass | Get Active Guidance () | Returns the Guidance object for the active task. Default task is DP operation. |
| Observer.lvclass | Get Active Observer () | Returns the Observer object for the active task. Default task is DP operation. |
| Void | Measurement Substitution (Void) | Substitutes the angle measured state with the ones from MTi. |
| Void | Read Configuration File () | Initializes the invoker and makes the system ready for execution. |
| 4 1D Array of Double | Read FPGA FIFO () | Reads the FIFO stack on the NI compactRIO and return arrays with the sensor values. NB! This method is only to be used together with a NI compactRIO unit. Use “Read ROV String_SIL” for non NI compactRIO setup. Return values is: - MRU - MT_DATA - Signals out - Dvl_m |
| 2 1D Array of Double | Read ROV String_SIL (String ROV string, String NAVI string) | Reads the sensor strings and returns the sensor values. This is a substitution for the Read FPGA FIFO () for setup with no NI compactRIO setup. Return Values is: - Signals out - Dvl_m |
| Void | Signal Processing (1D Array of Double MRU, 1D Array of Double MT_DATA, 1D Array of Double Signals in, 1D Array of Double dvl_m) | Translates the sensor values into position and velocity measurements. The Sensors health is checked. State variables are set in the invoking vessel object. |
| Void | Store Active Altitude Observer (Observer.lvclass observer) | Replaces the Altitude Observer object for the active task. Updates will then be carried on in the system. |
| Void | Store Active Controller (Controller.lvclass controller) | Replaces the Controller object for the active task. Updates will then be carried on in the system. |
| Void | Store Active Guidance (Guidance.lvclass guidance) | Replaces the Guidance object for the active task. Updates will then be carried on in the system. |
| Void | Store Active Observer (Observer.lvclass observer) | Replaces the Observer object for the active task. Updates will then be carried on in the system. |
| Void | Write to cRIO () | Create and write the telebuf to the NI compactRIO FIFO stack. NB! To not use this unless a NI compactRIO unit is connected. Use “Write to cRIO_SIL” instead. |
| 1D Array of UInt8 | Write to cRIO_SIL () | Create the telebuf and send it as output. This is a substitute for the “Write to cRIO”. |

Supervisor.lvclass

Labview Object.lvclass
	∟ Supervisor.lvclass

Implemented by Espen Tolpinrud.

The Supervisor object works as a communication administrator and execution flow manager.

Constructor Summary

| Name | Note |
| --- | --- |
| Supervisor() | Default constructor to create a supervisor object. |

Field Summary

| Data Type | Name | Note |
| --- | --- | --- |
| Boolean | Auto Depth | Used to give message about running auto depth in joystick control. |
| Boolean | Auto Depth Tele | Used to give message about running auto depth in joystick control. Message from ROV control panel. |
| Boolean | Auto Heading | Used to give message about running auto heading in joystick control. Message from Joystick. |
| Boolean | Auto Heading Tele | Used to give message about running auto heading in joystick control. Message from ROV control panel. |
| Cluster of { Boolean, Int8, Double, Double, Double, Double, Double} | Altitude | Cluster containing: - Altimeter Active (Boolean) - Beam Select (Int8) - Measured (Double) - Estimated (Double) - Desired (Double) - Previous Desired (Double) - Approximated (Double) |
| Cluster of { Boolean, Boolean, Boolean, Int32 } | Altitude Cluster | Cluster containing: - Use Integrator (Boolean) - Use FeedForward (Boolean) - Use Reference Model (Boolean) - Desired Altitude |
| Cluster of { 4x 1D Array of Double} | Attitude | Cluster containing: - Theta Measured (1D Array of Double) - Theta Estimated (1D Array of Double) - Omega Measured (1D Array of Double) - Omega Estimated (1D Array of Double) |
| Cluster of { Boolean} | Attitude Cluster | Cluster containing: - Initialize (Boolean) |
| Cluster of { Int32, Boolean, Boolean, Boolean, Boolean} | Camera Control | Cluster containing: - Camera Number (Int32) - Camera Up (Boolean) - Camera Down (Boolean) - Camera Left (Boolean) - Camera Right (Boolean) |
| Cluster of { 4x Boolean} | Collecting Unit | Contains: - In (Boolean) - Out (Boolean) - Rotate CW (Boolean) - Rotate CCW (Boolean) |
| UInt16 | Control Mode | Selector for the different control modes the vessel can operate in during Joystick mode. |
| Boolean | Control System Active | Flag used to determine if the control loop is to be run. |
| UInt16 | Coordinate System | Selector for the different coordinate reference systems the vessel can operate in. |
| Cluster of {1D Array of Double, 1D Array of Double, 1D Array of Double, 1D Array of Double} | Desired | Desired State values from the vessel. |
| 1D Array of Double | Dvl data | Sensor values from the Doppler Velocity Log. |
| String | Error Msg | Contains an Error Message from the system, if everything is OK, it should say “OK”. |
| Cluster of {1D Array of Double, 1D Array of Double, 1D Array of Double, 1D Array of Double} | Estimated | Estimated State values from the vessel. |
| Boolean | Exit Flag | Change when user interface demands it |
| UInt16 | Frame Mode | Selector for the different frame modes the vessel can operate in during joystick mode |
| Cluster of { Boolean, Boolean, Int8, Boolean, Boolean, Cluster of {Double, Double, Double, Double}, Cluster of {Boolean, 1D Array of Double}} | Guidance Cluster | Cluster contains: - Set Origin (Boolean) - New WP (Boolean) - Types of Change (Int8) - Make Change (Boolean) - Optimal Heading (Boolean) - New SetPoint - North - East - Depth - Heading - Update Reference - Mode Change - Current Position |
| Int32 | Iteration Number | The iteration number for the control loop |
| Cluster of {Cluster of {Double, Double, Double}, Cluster of {Double, Double, Double}} | Joystick Axis | Cluster with joystick axis inputs. Contains: - Axis Translation - X-Axis - Y-Axis - Z-Axis - Axis Rotation - X-Axis - Y-Axis - Z-Axis |
| Cluster of {Boolean, Boolean, Boolean, Boolean, Boolean, Boolean, Boolean, Boolean, Boolean, Boolean, Boolean, Boolean, Int32} | Joystick Commands | Cluster with joystick button inputs. Contains: - Button 1 (Boolean) - Button 2 (Boolean) - Button 3 (Boolean) - Button 4 (Boolean) - Button 5 (Boolean) - Button 6 (Boolean) - Button 7 (Boolean) - Button 8 (Boolean) - Button 9 (Boolean) - Button 10 (Boolean) - Button 11 (Boolean) - Button 12 (Boolean) - POV Direction (Int32) |
| Cluster of { 2 Double Matrix, 2 Boolean} | Kalman Filter Observer Tuning | Contains new tuning values for the Kalman Filter observer. Elements are: - R (Double Matrix) - Q (Double Matrix) - Update R (Boolean) - Update Q (Boolean) When the Booleans are set to true the corresponding matrices in the Non Linear PID object are updated to contain the values from the matrices in this cluster. |
| Cluster of {Boolean, Boolean, Boolean, Boolean, Boolean, Boolean, Int32, Int32} | Light Settings | Cluster with Light control inputs. Contains: - Light 1 - Light 2 - Light 3 - Light 4 - HMI 1 - HMI 2 - Light Intensity 1 - Light Intensity 2 |
| Cluster of {Boolean, Boolean, Boolean, Boolean, Boolean, Boolean, Boolean, Boolean, Boolean, Boolean, Boolean } | Manipulator Control | Cluster with Manipulator control inputs. Contains: - Power Switch (Boolean) - Upper Arm Up (Boolean) - Upper Arm Down (Boolean) - Lower Arm Up (Boolean) - Lower Arm Down (Boolean) - Arm Left (Boolean) - Arm Right (Boolean) - Claw Open (Boolean) - Claw Close (Boolean) - Rotate Clockwise (Boolean) - Rotate Counterclockwise (Boolean) |
| Cluster of {1D Array of Double, 1D Array of Double, 1D Array of Double, 1D Array of Double} | Measured | Measured State values from vessel. |
| 1D Array of Single | MT_DATA | Contains values from the MT sensor. Used in Attitude Observer. |
| Boolean | MTi Active | Flag to determine if MTi Observer shall run |
| Boolean | MTi Gyro | Flag to determine if MTi Gyro sensor shall be used. |
| Boolean | New WP List | Flag to notify about new WP List. |
| Cluster of { 4 Double Matrix, 4 Boolean} | Non Linear PID Tuning | Contains new tuning values for the Non Linear PID Controller. Elements are: - KP (Double Matrix) - KD (Double Matrix) - KI (Double Matrix) - KA (Double Matrix) - Update KP (Boolean) - Update KD (Boolean) - Update KI (Boolean) - Update KA (Boolean) When the Booleans are set to true the corresponding matrices in the Non Linear PID object are updated to contain the values from the matrices in this cluster. |
| Boolean | Only New HiPAP | Flag to determine if only new HiPAP measurements are to be sent to the observer. |
| UInt16 | Operation Type | Selector which keeps track of which mode is active for the vessel the supervisor object belongs to. |
| Cluster of {Double, Double} | Origin | Represents the UTM coordinates for a desired origin. Contains: - North (Double) - East (Double) |
| Cluster of { 10x Boolean} | Power Instruments | Contains - HPR (Boolean) - Sonar 1 (Boolean) - Sonar 2 (Boolean) - Doppler (Boolean) - Laser (Boolean) - KRAFT (Boolean) - Pan/Tilt (Boolean) - Transponder (Boolean) - Still Cam (Boolean) - HD Cam (Boolean) |
| Boolean | Reset Controller | Flag to reset integrators in the controllers. |
| Boolean | Reset Observer | Flag to reset the values in the observers. |
| Boolean | Reset ROV | Flag to set the desired position to current estimated position. |
| Boolean | Reset SP | Flag used to notify if signal processing is to be reset. |
| 1D Array of Double | RPM | RPM values for the vessel the supervisor object belongs to. |
| Cluster of {1D Array of Int32, 1D Array of Boolean} | Sensor Status | Contains: - Sensor Drop Out (1D Array of Int32) - Sensor Health (1D Array of Boolean) |
| 2D Array of Double | Table of WP | An array with the waypoints to be used in tracking. |
| 1D Array of Double | Tau_scaled | A percentage representation of the desired thrust in each Degree of Freedom. |
| 1D Array of String | Thruster Names | A string array with the thruster names. |
| Cluster of{Cluster of {Double, Double, Double, Double, Double, Double, Double, Double}, 2D Array of Double, Cluster of { Double, Double, Double, Double, Double, Double, Double, Double, Double}, UInt16, Boolean, Boolean, Boolean, Boolean} | Tracking Cluster | A Cluster with tracking performance settings. Contains: - Performance Behavior - % Velocity max lin to exp (Double) - % Velocity zero lin to exp (Double) - % Total distance at max velocity (Double) - Max Angular Velocity (Double) - Time to max linear velocity (Double) - Min distance at max velocity (Double) - Max linear velocity (Double) - Time to max angular velocity (Double) - Table of WP (2D Array of Double) - Tracking Options - Superior Bound (Double) - Inferior Bound (Double) - Duration No operation period between motions (Double) - Tolerance of initial angle (Double) - Distance to activation waiting function (Double) - Distance to deactivation waiting function (Double) - (2) Fixed Heading (Double) - (3) PoI N (Double) - (3) PoI E (Double) - Heading Mode (UInt16) - Start (Boolean) - Use Position (Boolean) - Pause (Boolean) - Abort (Boolean) |
| Boolean | Update Ports | A Boolean to notify the system about new port configurations. |
| 1D Array of Cluster of {Int16, Int8, Int8, Int8, Int8} | Updated Port Configurations | Array with communication information used to communicate with the NI compactRIO. The cluster contains the fields: - Baud Rate (Int16) - Data Bits (Int8) - Parity (Int8) - Stop Bits (Int8) - Flow Control (Int8) |
| Boolean | Use Altitude Guidance | A Boolean to notify the system about using altitude guidance. |
| Boolean | Use Chair Joystick | A Boolean to notify the system about using the chair joystick instead of the one connected to the host pc. Yields only for 30k. |

Method Summary

| Data Type of Return | Name with input arguments | Note |
| --- | --- | --- |
| TCP Network Reference | Receive TCP Message (TCP Network Reference ref) | Receive run-time commands through the TCP protocol. Usually from a user interface. |
| TCP Network Reference | Receive TCP Message Joystick (TCP Network Reference ref) | Receive Joystick commands through the TCP protocol and fills out Joystick Axis and Joystick Commands. Usually from a user interface. |
| TCP Network Reference | Send TCP Message (TCP Network Reference ref) | Sends status messages through the TCP protocol, usually to a user interface. |

Method Details

Receive TCP Message (TCP Network Reference ref)

This method is used to listen for new inputs from the user interface. Each message contains a code and a message body.

| Code | Message Body | Note |
| --- | --- | --- |
| 1: | 1 | Exit message to the system. |
| 2: | Double North,Double East,Double Depth,Double Heading | Notification of new DP position. The coordinates should be given in NED coordinates with Heading in radians. |
| 3: | Integer nWP,Double wp1index,Double wp1n,Double wp1e,Double wp1d;…;Double wpNindex,Double wpNn,Double wpNe,Double wpNd; | Notification of tracking operation. The whole waypoint list is sent starting with the number of waypoints, N, and then continuing with N sets of NED coordinates. |
| 4: | 3charString code,Integer value | Message for Manipulator control on the ROV. The code has to consist of 3 and only 3 characters! The valid codes are: - PSM (Power Switch Manipulator) - UA (Upper Arm) - U (Up) - D (Down) - LA (Lower Arm) - U (Up) - D (Down) - MA (Manipulator Arm) - R (Right) - L (Left) - CL (Claw) - C (Close) - O (Open) - AR (Arm Rotation) - L (Counterclockwise) - R (Clockwise) The integer values should be either 0 (for false) or 1 (for true). The true condition can however be set to any integer not equal to 0. |
| 5: | 2charString code,Integer value | Message for Camera settings on the ROV. The code has to consist of 2 and only 2 characters! The valid codes are: - CN (Camera Number) - CU (Camera Up) - CD (Camera Down) - CL (Camera Left) - CR (Camera Right) The integer value should be for all codes except CN be 0 (for false) or 1 (for true). For CN the integer value should be a whole number. |
| 6: | 2charString code,Integer value | Message for Light settings on the ROV. The code has to consist of 2 and only 2 characters! The character at the back must be an integer. The valid codes are: - L (Light) - 1 - 2 - 3 - 4 - H (Hydrargyrum medium-arc iodide light) - 1 - 2 - I (Light Intensity) - 1 - 2 The integer value should for all codes except for I be 0 (for false) or 1 (for true). For I the value should be a whole number between 0 and 127. |
| 7: |  | Joystick Command String. (See details below) |
| 8: | 3charString,Double value | Message for changing options for Tracking |
| 9: | 1charString,Integer | Message for Pause or Abort Tracking. |
| 10: | 8x[Integer PortNumber,Integer BaudRate,Integer DataBits,Integer Parity,Integer StopBit,Integer FlowControl;] | Message for configuration of port connections between the system and the compactRIO. 8 port configurations are sent from the user interface. |
| 11: | Integer value | Notification to the system to set new origin point. The integer value should be 0 (for false) or 1 (for true). |
| 12: | Integer value | Message used to switch between using estimated and measured values in the controller. |
| 13: | Integer value | Message used to switch frame mode during joystick operations. The integer value points to a selector position. |
| 14: | Integer value | Message used to switch control mode during joystick operations. The integer value point to a selector position. |
| 15: | Integer value | Message to notify the system to start the control loop. The integer value should be 0 (for false) or 1 (for true). |
| 16: | Integer value | Message to notify the system to reset the observer. The integer value should be 0 (for false) or 1 (for true). |
| 17: | Integer value | Message to notify the system to reset the controller. The integer value should be 0 (for false) or 1 (for true). |
| 18: | Integer value | Message to notify the system to reset the ROV. The integer value should be 0 (for false) or 1 (for true). The controller is also reset by this call. The desired position is set to be the estimate. |
| 19: | 3charString,[1charString:1charString] [MatrixValuesString] | Message to notify the system to update the tuning matrices for a controller or observer. Which module to be updated, is given in the message. |
| 20: | Integer Value | Message to notify the system to activate/deactivate the MTi. The integer value should be 0 (for false) or 1 (for true). |
| 21: | Integer Value | Message to notify the system to activate/deactivate the MTi Gyro. The integer value should be 0 (for false) or 1 (for true). |
| 22: | Integer Value | Message to notify the system to activate/deactivate the auto tuning of Omega used by the reference model. The integer value should be 0 (for false) or 1 (for true). |
| 23: | Empty message body | Activates the joystick mode in force frame. Used as a fail-safe mechanism. |
| 24: | Integer Value | Message to notify the system to activate/deactivate the use of the joystick on the chair (for 30k only). The integer value should be 0 (for false) or 1 (for true). |
| 25: | Integer Value | Message to notify the system to activate/deactivate the HPR. The integer value should be 0 (for false) or 1 (for true). |
| 26: | Integer Value | Message to notify the system to activate/deactivate the Sonar 1. The integer value should be 0 (for false) or 1 (for true). |
| 27: | Integer Value | Message to notify the system to activate/deactivate the Sonar 2. The integer value should be 0 (for false) or 1 (for true). |
| 28: | Integer Value | Message to notify the system to activate/deactivate the Doppler Velocity Log. The integer value should be 0 (for false) or 1 (for true). |
| 29: | Integer Value | Message to notify the system to activate/deactivate the Laser. The integer value should be 0 (for false) or 1 (for true). |
| 30: | Integer Value | Message to notify the system to activate/deactivate the KRAFT Manipulator Arm Control. The integer value should be 0 (for false) or 1 (for true). |
| 31: | Integer Value | Message to notify the system to activate/deactivate the Pan/Tilt. The integer value should be 0 (for false) or 1 (for true). |
| 32: | Integer Value | Message to notify the system to activate/deactivate the Transponder. The integer value should be 0 (for false) or 1 (for true). |
| 33: | Integer Value | Message to notify the system to activate/deactivate the Still Camera function. The integer value should be 0 (for false) or 1 (for true). |
| 34: | Integer Value | Message to notify the system to activate/deactivate the HD Camera. The integer value should be 0 (for false) or 1 (for true). |
| 35: | 2charString,Integer Value | Message to notify the system to activate/deactivate the Collecting Unit functions. The integer value should be 0 (for false) or 1 (for true). Codes are: - OU – Out - IN – In - CC – Counter Clockwise - CW - ClockWise |
| 36: | Integer Values | Message to notify the system to activate/deactivate the “Only New HiPAP”. The integer value should be 0 (for false) or 1 (for true). |
| 37: | Integer Value | Message to notify the system to reset the Signal Processing. The integer value should be 0 (for false) or 1 (for true). |
| 38: | Empty Message body | Notifies the system to send a list of all available controllers and observers to the user interface. |

Receive TCP Message Joystick (TCP Network Reference ref)

This method is used to receive joystick commands from the user interface. This has been given its own method due to frequent message receiving. By running this method in a separate thread concurrent with Receive TCP Message () no user inputs are ignored.

| Code | Message Body | Note |
| --- | --- | --- |
| 7: | Integer value,Double X,Double Y, Double Z, Double ZRot,Integer Buttons,Integer POV | The integer value is a numeric representation of a Boolean value of whether or not the joystick is in use or not. X is the X-Axis position, Y is the Y-Axis position, Z is the Z-Axis position and the ZRot is the Z-Axis Rotation. The integer Buttons is a numeric representation of a Boolean array and has a value of i=0k2i where k is the number of buttons on the joystick. Each button combination then has its own unique value and no button pressed is set to 0. The POV integer gives the direction angle in degrees from 0 to 360 with step of 45 degrees. No direction is defined as -1. |

Send TCP Message (1D Array of TCP Network Reference ref)

This method builds and sends a string of data to the user interface. There are separate TCP Network References for each type of data set to be sent.

The different data sets are:

- Position states

- Velocity states

- Thruster values and RPMs

- Altitude and dvl sensor

- MTi data (Not implemented yet)

- Other sensors can be added.

Thrust Allocation.lvclass

LabView Object.lvclass
	∟ Thrust Allocation.lvclass

Implemented by Viktor Berg.

Constructor Summary

| Name | Note |
| --- | --- |
| Thrust Allocation() | Default constructor to create an instance of the class. |

Field Summary

| Data Type | Name | Note |
| --- | --- | --- |
| 1D Array of Thruster.lvclass | Array Thrusters | Array of the thruster objects that belongs to the invoking thrust allocation object. |
| Matrix | Thrust Allocation Matrix | A matrix with thruster impact in each Degree of Freedom. Each row represents Degree of Freedom, and the columns are the thrusters. Also called T. |
| Matrix | Thrust Coefficients Matrix | A Matrix with the relationship between thrust in force and thruster RPMs. Also called K. |

Method Summary

| Data Type of Return | Name with input arguments | Note |
| --- | --- | --- |
| Vessel.lvclass | Calculate Thrust(Vessel.lvclass vessel) | Converts the desired thruster forces and torques to thruster RPMs with the expression u=K-1T†τ |
| Void | Insert Thruster(Thruster.lvclass thruster, Int32 Index) | Insert a new thruster object in the Array Thruster. The thruster impact in each Degree of Freedom is calculated and added to the Thrust Allocation Matrix. |
| Void | Thrust Coefficients (1D Array of Double rpm) | The method calculates the Thrust Coefficients to be used by the Calculate Thrust method. |
| Void | Write Col in Thrust Allocation Matrix ( | This is a private method for the Thrust Allocation object and is used to add a column in the Thrust Allocation Matrix. |

Thruster.lvclass

LabView Object.lvclass
	∟ Thruster.lvclass

Implemented by Espen Tolpinrud.

Constructor Summary

| Name | Note |
| --- | --- |
| Thruster() | Default constructor to create an instance of the class. |
| Thruster (1D Array of Double input, String Type, String Name) | Constructor that fully initialize the object. |

Field Summary

| Data Type | Name | Note |
| --- | --- | --- |
| 1D Array of Double | Coefficients | Array with coefficients for the relationship between Force and RPM. Contains both for positive and negative RPM, and is made for second order relationship. |
| Double | Max RPM | A matrix with thruster impact in each Degree of Freedom. Each row represents Degree of Freedom, and the columns are the thrusters. Also called T. |
| String | Name | A Matrix with the relationship between thrust in force and thruster RPMs. Also called K. |
| 1D Array of Double | Position Vector | The position vector represents the thruster position in body coordinates. First index is x, second y, third z, fourth rotation about z-axis, fifth rotation between x-y plane and z-axis. This is similar to spherical coordinates. |
| 1D Array of Double | Rotation Vector | If thruster is Azimuth, this vector tells which axis to rotate about. |
| UInt16 | Thruster Type | Selector of thruster type. Yields for fixed and Azimuth. |

Method Summary

None

Sensor.lvclass

LabView Object.lvclass
	∟ Sensor.lvclass

Implemented by Espen Tolpinrud.

This class may be unnecessary at the moment. Removal of the class is suggestible.

Constructor Summary

| Name | Note |
| --- | --- |
| Sensor() | Default constructor to create an instance of the class. |
| Sensor (1D Array of String input) | Constructor that fully initialize the object. |

Field Summary

| Data Type | Name | Note |
| --- | --- | --- |
| String | ID | The Sensor ID |
| String | Name | The Name of the sensor |
| String | PosX | The x position of the sensor from Center of Origin |
| String | PosY | The y position of the sensor from Center of Origin |
| String | PosZ | The z position of the sensor from the Center of Origin |
| String | RotX | The x rotation of the sensor |
| String | RotY | The y rotation of the sensor |
| String | RotZ | The z rotation of the sensor |
| String | Type | The type of the sensor. |

Method Summary

| Data Type of Return | Name with input arguments | Note |
| --- | --- | --- |
| Boolean | Check Health (Int32 Drop Out) | Checks the health of the invoking sensor. NB! To be developed further. |
| Void | WriteAllData (1D Array of String) | Fully Initialize the object. Is used in Sensor(1D Array of String input) |

Observer.lvclass

LabView Object.lvclass
	∟ Observer.lvclass

Implemented by Espen Tolpinrud.

This is a template or abstract class for all observers and should never be used stand alone.

Constructor Summary

| Name | Note |
| --- | --- |
| Observer() | Default constructor to create an instance of the class. |
| Observer(String input) | Constructor that sets the name of the Observer object. |

Field Summary

| Data Type | Name | Note |
| --- | --- | --- |
| String | Observer Name | The name of the observer |

Method Summary

| Data Type of Return | Name with input arguments | Note |
| --- | --- | --- |
| Vessel.lvclass | EstimateStates (Vessel.lvclass vessel) | A method that every child of Observer.lvclass must override. The method calculates the estimated states for the system. |

Kalman Filter.lvclass

LabView Object.lvclass
	∟ Observer.lvclass
		∟ Kalman Filter.lvclass

Implemented by Viktor Berg.

Constructor Summary

| Name | Note |
| --- | --- |
| Kalman Filter() | Default constructor to create an instance of the class. |
| Kalman Filter (Vessel.lvclass vessel) | Constructor that initializes PHI, GAMMA, and DELTA. |

Field Summary

| Data Type | Name | Note |
| --- | --- | --- |
| Matrix | DELTA | Sector matrix for Kalman Filter. |
| Matrix | GAMMA | Sector matrix for Kalman Filter. |
| Matrix | PHI | Sector matrix for Kalman Filter. |
| Matrix | Q | Tuning matrix for Kalman Filter. |
| Matrix | R | Tuning matrix for Kalman Filter. |

Method Summary

| Data Type of Return | Name with input arguments | Note |
| --- | --- | --- |
| 2D array of Int32 | Drop Out Converter (1D Array of Int32 Drop Out) | Transform the drop out vector to a measurement matrix. May not be needed anymore. |
| Vessel.lvclass | EstimateStates (Vessel.lvclass vessel) | Estimate states by using predictors and Kalman Filter equations. |
| 3 1D Array of Double | Init Observed Values | Set up the measured values to be given the Kalman Filter. The output is: - y (1D Array of Double) - y_last (1D Array of Double) - tau (1D Array of Double) Note to developer: May need revising. |
| 5 Matrix | Read Q and R | Returns DELTA, GAMMA, PHI, Q and R. |

Methods Details

EstimateStates(Vessel.lvclass vessel)

//TODO: Insert equations used to calculate the estimated states.

Kalman Filter Altitude.lvclass

LabView Object.lvclass
	∟ Observer.lvclass
		∟ Kalman Filter.lvclass
			∟ Kalman Filter Altitude.lvclass

Implemented by Espen Tolpinrud.

Constructor Summary

| Name | Note |
| --- | --- |
| Kalman Filter Altitude() | Default constructor to create an instance of the class. |

Method Summary

| Data Type of Return | Name with input arguments | Note |
| --- | --- | --- |
| Vessel.lvclass | EstimateStates (Vessel.lvclass vessel) | Estimates the altitude of the vessel. |

Method Details

EstimateStates (Vessel.lvclass vessel)

// TODO: Insert equations for estimating the states.

Passive Observer.lvclass

LabView Object.lvclass
	∟ Observer.lvclass
		∟ Passive Observer.lvclass

Implemented by Espen Tolpinrud.

Constructor Summary

| Name | Note |
| --- | --- |
| Passive Observer() | Default constructor to create an instance of the class. |

Field Summary

| Data Type | Name | Note |
| --- | --- | --- |
| Matrix | Aw | System matrix for wave filter. |
| Matrix | Cw | Measurement matrix for wave filter. |
| Matrix | Delta | Wave damping matrix. Typical values are ζni=1.0 |
| Matrix | K1 | Tuning matrix for wave filter. Typical values are K1iωoi=-2ζni-λiωciωoi , ∀i=1..6 K1i+6ωoi=2ωoiζni-λi,∀i=1..6 |
| Matrix | K2 | Tuning matrix for observer. Typical values are K2i=ωci |
| Matrix | K3 | Tuning matrix for bias estimator. Typical values K3=0.1K4 |
| Matrix | K4 | Tuning matrix for observer. |
| 1D Array of Double | Lambda | Tuning variables to determine the notch. Typical value is λi=0.1 |
| Matrix | Omega | The diagonal elements are the frequency of the time periods for the waves. Ω=diag2πTi,∀i=1..6 where typical values for Ti=5..20 seconds |
| 1D Array of Double | Omega_cutoff | The cutoff frequency for the wave period. Must be larger than Omega. |
| Matrix | T | Time constants for the bias estimator. |

Method Summary

| Data Type of Return | Name with input arguments | Note |
| --- | --- | --- |
| 1D Array of Double | Bias Estimator (1D Array of Double y, Double dt) | Estimates the bias. |
| Vessel.lvclass | EstimateStates (Vessel.lvclass vessel) | Estimates the states for the system using the equations for the passive observer. Have both bias estimator and wave filter. Wave filter is optional. |
| Void | Initialize Matrices () | Initializes the system matrices using tuning expressions. |
| 1D Array of Double | Wave Estimator (1D Array of Double y, Double dt) | Wave filter for the passive nonlinear observer. |

Method Details

Bias Estimator (1D Array of Double y, Double dt)

EstimateStates (Vessel.lvclass vessel)

Initialize Matrices ()

Wave Estimator (1D Array of Double y, Double dt)

Adaptive Passive Observer.lvclass

LabView Object.lvclass
	∟ Observer.lvclass
		∟ Passive Observer.lvclass
			∟ Adaptive Observer.lvclass

Implemented by Viktor Berg.

Constructor Summary

| Name | Note |
| --- | --- |
| Adaptive Observer() | Default constructor to create an instance of the class. |

Field Summary

| Data Type | Name | Note |
| --- | --- | --- |
| Matrix | Gamma wave |  |
| Matrix | K1h |  |
| Matrix | K2h |  |
| Matrix | K2l |  |
| Matrix | K3h |  |
| Matrix | K3l |  |
| Matrix | K4h |  |
| Matrix | K4l |  |
| Matrix | Omega h |  |
| Matrix | Omega l |  |
| Matrix | Tf |  |

Method Summary

| Data Type of Return | Name with input arguments | Note |
| --- | --- | --- |
| Matrix | Adaptive Law (…) |  |
| 1D Array of Double | Bias Estimator |  |
| Vessel.lvclass | EstimateStates (Vessel.lvclass vessel) | Estimates the states. |
|  | Filter |  |
| Void | Initialize Matrices () |  |
| 1D Array of Double | Wave Estimator |  |

Guidance.lvclass

Labview Object.lvclass
	∟ Guidance.lvclass

Implemented by Espen Tolpinrud.

This is a template or abstract class for all guidance modules in the control system.

Constructor Summary

| Name | Note |
| --- | --- |
| Guidance() | Default constructor to create an instance of the class. |

Field Summary

| Data Type | Name | Note |
| --- | --- | --- |
| Matrix | Delta | Tuning matrix for filter based reference model (Outgoing) |
| Matrix | Omega | Tuning matrix for filter based reference model (Outgoing) |

Method Summary

| Data Type of Return | Name with input arguments | Note |
| --- | --- | --- |
| Vessel.lvclass | Calculate Desired States (Vessel.lvclass vessel) | Calculates the desired position and velocity for the vessel. This method must be implemented by all children of this class. |

Altitude Guidance.lvclass

Labview Object.lvclass
	∟ Guidance.lvclass
		∟ Altitude Guidance.lvclass

Implemented by Espen Tolpinrud.

Guidance Law for altitude.

Constructor Summary

| Name | Note |
| --- | --- |
| Altitude Guidance() | Default constructor to create an instance of the class. |

Field Summary

| Data Type | Name | Note |
| --- | --- | --- |
| Double | K_ff | Tuning value for feed forward term |
| Double | K_i | Tuning value for integrator term |
| Double | K_p | Tuning value for proportional term |

Method Summary

| Data Type of Return | Name with input arguments | Note |
| --- | --- | --- |
| - | Altitude GL | Private function for Altitude Guidance, used by Calculate Desired States |
| Vessel.lvclass | Calculate Desired States (Vessel.lvclass vessel) | Calculates the desired altitude the vessel. |

DP Guidance.lvclass

Labview Object.lvclass
	∟ Guidance.lvclass
		∟ DP Guidance.lvclass

Implemented by Espen Tolpinrud.

Constructor Summary

| Name | Note |
| --- | --- |
| DP Guidance() | Default constructor to create an instance of the class. |

Field Summary

| Data Type | Name | Note |
| --- | --- | --- |
| Double | Criterium for Optimal Heading | Tuning value used in the optimal heading method (Not implemented) (Can be removed?) |
| 1D Array of Double | Position Reference |  |
| Cluster of {1D Array of Double, 1D Array of Double, 1D Array of Double, 1D Array of Double} | Previous Kinematics |  |
| 1D Array of Double | Restart Criteria | Tuning for the feedback method. Used to determine when to restart desired position updates. |
| 1D Array of Double | Wait Criteria | Tuning for the feedback method. Used to determine when to wait for the vessel to catch up to the reference. |

Method Summary

| Data Type of Return | Name with input arguments | Note |
| --- | --- | --- |
| Vessel.lvclass | Calculate Desired States (Vessel.lvclass vessel) | Calculates the desired position and velocity for the vessel. |
| 3 1D Array of double, Boolean | Feedback (Vessel.lvclass vessel, Boolean reset_in, Boolean use_estimated_values) | Calculate the next desired step if the vessel is close enough to the current desired step. If not, the reference waits for the vessel to catch up. |

Joystick Guidance.lvclass

Labview Object.lvclass
	∟ Guidance.lvclass
		∟ Joystick Guidance.lvclass

Implemented by Espen Tolpinrud.

Constructor Summary

| Name | Note |
| --- | --- |
| Joystick Guidance() | Default constructor to create an instance of the class. |

Field Summary

| Data Type | Name | Note |
| --- | --- | --- |
| UInt16 | Control Mode | Selector of what control mode the joystick input is set for. Modes are Position, Velocity and Thrust. |
| UInt16 | Control Mode Previous | Selector of what control mode the joystick input is set for from last time step |
| 1D Array of Double | Eta Set | A copy of desired position. Helps to restrain depth changes due to uncontrolled pitch movement while driving. |
| Uint16 | Frame Mode | Selector of what frame mode the joystick input is set for. Modes are Body, NED and cylindrical. |
| UInt16 | Frame Mode Previous | Selector of what frame mode the joystick input is set for from last time step. |
| Cluster of {Boolean, Boolean, Boolean, Boolean} | Inside_Dead_Zone | A cluster of Booleans. A cluster element is true if the input in the corresponding direction or rotation is zero i.e. inside the dead zone. |
| Cluster of {Boolean, Boolean, Boolean, Boolean} | Inside_Dead_Zone_Previous | A cluster of Booleans. A cluster element is true if the input in the corresponding direction or rotation is zero i.e. inside the dead zone from last time step. |
| Int32 | Loop count | The iteration number for the control system. |
| Double | Radius | The radius used in cylindrical frame mode. |
| Double | Radius Initially | The initially radius in cylindrical frame mode. |
| Boolean | Set Origin | A Boolean used to notify whether or not to set origin. |
| Boolean | Set Turn Point | A Boolean used to notify whether or not to set turn point. |
| Double | Step Depth | A standard step in depth direction. Usually used to make small corrections in the direction by button inputs or POV direction stick. |
| Double | Step Heading | A standard step in heading angle. Usually used to make small corrections on the angle by button inputs or POV direction stick. |
| Double | Step Surge | A standard step in surge direction. Usually used to make small corrections in the direction by button inputs or POV direction stick. |
| Double | Step Sway | A standard step in sway direction. Usually used to make small corrections in the direction by button inputs or POV direction stick. |
| Double | Theta | Used in cylindrical frame mode |
| Cluster of {19 Boolean} | VI Flow Criteria | The cluster is updated on each time step by the Compute Conditions method. The cluster contains - (1) CylinderMode AND (2) - (2) Out of DeadZone-Z-Axis Rot AND (3) - (3) Change ControlMode OR (10) - (4) Out of DeadZone XY-Plane - (5) Out of DeadZone XY-Plane OR Set Origin Point - (6) Set Origin Point - (7) Deactivating CylinderMode OR Initialize - (8) Deactivate CylinerMode - (9) Initialize - (10) Activating CylinderMode - (11) Set Turn pos OR (10) - (12) (11) AND CylinderMode - (13) (11) OR (1) - (14) (1) OR (16) - (15) Activating OR Deactivating CylinderMode - (16) (15) OR Initialize - (17) (5) OR (16) - (18) Out of DeadZone Z-Axis Rot OR (17) - (19) Out of DeadZone Z-Axis Rot |

Method Summary

| Data Type of Return | Name with input arguments | Note |
| --- | --- | --- |
| Vessel.lvclass | Calculate Desired States (Vessel.lvclass vessel) | Calculates the desired position and velocity for the vessel. Input from user through joystick is used to generate the desired states. |
| Void | Compute Condition (Double X-Axis, Double Y-Axis, Double Z-Axis, Double Z-Rotation, Int32 Loop nr, Boolean First) | Compute the Boolean values in the cluster VI Flow Criteria. |
| 1D Array of Double | Cylinder Kinematics Modification (1D Array of Double Max Velocity, Double Radius) | Modify the max velocity to cylinder coordinates. |
| 1D Array of Double, Double, Double | Eta Set Update (1D Array of Double eta, Boolean condition, Boolean CylinderMode) | Updates the position reference used to maintain desired depth and heading. |
| Boolean | Initialize Check (Int32 Loop Counter) | Checks if the system needs to be initialized or re-initialized. |

Tracking Guidance.lvclass

Labview Object.lvclass
	∟ Guidance.lvclass
		∟ Tracking Guidance.lvclass

Constructor Summary

| Name | Note |
| --- | --- |
| Tracking Guidance() | Default constructor to create an instance of the class. |
| Tracking Guidance (Matrix Omega, Matrix Delta) | Constructor which also sets the reference model tuning matrices. |

Field Summary

| Data Type | Name | Note |
| --- | --- | --- |
| Double | Acceptance Circle | Representation of the tolerance of position error in order to change way point. |
| Cluster | Previous Kinematics | States from last time step. |
| 1D Array of Double | Restart Criteria | Tuning for the feedback method. Used to determine when to restart desired position updates. |
| 1D Array of Double | Wait Criteria | Tuning for the feedback method. Used to determine when to wait for the vessel to catch up to the reference. |

Method Summary

| Data Type of Return | Name with input arguments | Note |
| --- | --- | --- |
| Vessel.lvclass | Calculate Desired States (Vessel.lvclass vessel) | Calculates the desired position and velocity for the vessel. Uses a list of waypoints to perform simple and naive tracking. (To be substituted at a later point) |
| 3 1D Array of Double | Feedback (Vessel.lvclass vessel, 1D Array of Double eta_ref, Boolean reset_in, Boolean Use Estimated Values) | Calculate the next desired step if the vessel is close enough to the current desired step. If not, the reference waits for the vessel to catch up. |

Tracking STRM Guidance.lvclass

Labview Object.lvclass
	∟ Guidance.lvclass
		∟ Tracking Guidance STRM.lvclass

Implemented by Espen Tolpinrud.

Constructor Summary

| Name | Note |
| --- | --- |
| Tracking Guidance STRM() | Default constructor to create an instance of the class. |

Field Summary

| Data Type | Name | Note |
| --- | --- | --- |
| Boolean | Abort | Boolean flag used to notify if tracking action is to be aborted. |
| Cluster of {19 Double} | Guidance Engine Parameters | Running variables for guidance engine. (May be removed) |
| UInt16 | Heading Mode | Selector of heading mode for tracking. |
| Boolean | Pause | Boolean flag used to notify if tracking is to be paused. |
| Cluster {8 Double} | Performance Behavior | Cluster of settings regarding ROV behavior in tracking. Values are: - % Velocity max lin to exp - % Velocity zero lin to exp - % Total dist at max velocity - % Max angular velocity - Time to max angular velocity - Min dist at max velocity - Max linear velocity - Time to max linear velocity |
| Double | psivar | Heading, used in variable heading case. |
| Boolean | Reset | Boolean flag used to notify if tracking action is to be reset. |
| Boolean | Start | Boolean flag used to notify if tracking action is to be started. |
| Cluster of {4 Double} | STRM | Cluster with values from the SynThetic Reference Model (STRM) |
| Boolean | Tracking Active | Boolean flag used to notify if tracking action is active. |
| Cluster {6 Boolean} | Tracking Engine Run-Time Logic | Cluster of Boolean flags used by the guidance engine. Values are: - Complete - Running - clrRMmemo - faststart - newcalc - aborting |
| Cluster {} | Tracking Options | Cluster with settings for the tracking. Values are: - (2) Fixed Heading - (3) PoI N - (3) PoI E - % Superior Bound - % Inferior Bound - Duration no operation period between motions - Tolerance of initial angle - Distance to activate waiting function - Distance to deactivate waiting function |
| Boolean | Use Depth | Boolean flag used to notify if tracking action is to use current depth. |
| Boolean | Waiting Function | Boolean flag used to notify if tracking action is to use waiting function. |

Method Summary

| Data Type of Return | Name with input arguments | Note |
| --- | --- | --- |
| Vessel.lvclass | Calculate Desired States (Vessel.lvclass vessel) | Calculates the desired position and velocity for the vessel. This method must be implemented by all children of this class. |
| Boolean | Check if Done () | Checks if complete and not running |
| Boolean | Reset Action () | If reset, set waiting function and use depth to false. |

Controller.lvclass

Labview Object.lvclass
	∟ Controller.lvclass

Implemented by Espen Tolpinrud.

A template or abstract class for controller modules in the control system.

Constructor Summary

| Name | Note |
| --- | --- |
| Controller() | Default constructor to create an instance of the class. |
| Controller (String name) | Constructor which sets the name of the controller. |

Field Summary

| Data Type | Name | Note |
| --- | --- | --- |
| String | Controller Name | The name of the controller, not much used. |

Method Summary

| Data Type of Return | Name with input arguments | Note |
| --- | --- | --- |
| Vessel.lvclass | Calculate Forces (Vessel.lvclass vessel) | Calculates the desired forces for the vessel. This method must be implemented on all children of this class. |

Joystick Controller.lvclass

Labview Object.lvclass
	∟ Controller.lvclass
		∟ Joystick Controller.lvclass

Implemented by Espen Tolpinrud.

Controller used to control the vessel with direct thrust input

Constructor Summary

| Name | Note |
| --- | --- |
| Controller() | Default constructor to create an instance of the class. |

Field Summary

| Data Type | Name | Note |
| --- | --- | --- |
| Cluster of {3 Double} | PID Gains Heading | Tuning values for PID controller for heading |
| Cluster of {3 Double} | PID Gains Depth | Tuning values for PID controller for depth |

Method Summary

| Data Type of Return | Name with input arguments | Note |
| --- | --- | --- |
| Vessel.lvclass | Calculate Forces (Vessel.lvclass vessel) | Gets joystick commands and sets up a desired thrust according to the input. Contains auto depth and auto heading as well. |

PID Controller.lvclass

Labview Object.lvclass
	∟ Controller.lvclass
		∟ PID Controller.lvclass

Implemented by Espen Tolpinrud.

Constructor Summary

| Name | Note |
| --- | --- |
| PID Controller() | Default constructor to create an instance of the class. |

Field Summary

| Data Type | Name | Note |
| --- | --- | --- |
| Matrix | K_D | Tuning matrix for the derivative term in the controller. |
| Matrix | K_I | Tuning matrix for the integral term in the controller. |
| Matrix | K_P | Tuning matrix for the proportional term in the controller. |

Method Summary

| Data Type of Return | Name | Note |
| --- | --- | --- |
| Vessel.lvclass | Calculate Forces (Vessel.lvclass vessel) | Calculates the desired forces for the vessel. This method must be implemented by all children of this class. |
| Void | Set Gains (3D Array of Double) | The tuning matrices are given through a 3D array and then set in their respective fields. K_P should be at the first index, followed by K_I in second. Last the K_D should be on the third index. |

Linear PID Controller.lvclass

Labview Object.lvclass
	∟ Controller.lvclass
		∟ PID Controller.lvclass
			∟ Linear PID Controller.lvclass

Implemented by Espen Tolpinrud.

Constructor Summary

| Name | Note |
| --- | --- |
| Linear PID Controller() | Default constructor to create an instance of the class. |
| Linear PID Controller (Matrix KP, Matrix KI, Matrix KD) | Constructor which gets and sets the tuning matrices. |

Method Summary

| Data Type of Return | Name with input arguments | Note |
| --- | --- | --- |
| Vessel.lvclass | Calculate Forces (Vessel.lvclass vessel) | Calculates the desired forces for the vessel with the expression τ=-(KPη+KI∫ηdt+KDη) |

Non Linear PID Controller.lvclass

Labview Object.lvclass
	∟ Controller.lvclass
		∟ PID Controller.lvclass
			∟ Non Linear PID Controller.lvclass

Implemented by Viktor Berg. Modified by Espen Tolpinrud to include optional Speed Controller.

Constructor Summary

| Name | Note |
| --- | --- |
| Non Linear PID Controller () | Default constructor to create an instance of the class. |
| Non Linear PID Controller (Matrix KP, Matrix KI, Matrix KD) | Constructor which gets and sets the tuning matrices. |

Field Summary

| Data Type | Name | Note |
| --- | --- | --- |
| 1D Array of Double | Int_limit | Used to check if the integral term shall integrate the error. If the error is larger than the Integral limit, zero is sent to the integrator. |
| Matrix | K_anti | Tuning matrix for the anti-wind up effect in the integral term. |
| Matrix | K_I_speed | Tuning matrix for the integral effect for the speed controller term |
| Boolean | Speed Controller | Flag to determine if to use speed controller term or not. |

Method Summary

| Data Type of Return | Name with input arguments | Note |
| --- | --- | --- |
| Vessel.lvclass | Calculate Forces (Vessel.lvclass vessel) | Calculates the desired position and velocity for the vessel. Similar to the linear PID but also include a Feed Forward term to include non-linear effects. |

Method Details

LQR Controller.lvclass

Labview Object.lvclass
	∟ Controller.lvclass
		∟ PID Controller.lvclass
			∟ Non Linear PID Controller.lvclass
				∟ LQR.lvclass

Implemented by Viktor Berg. Revised by Espen Tolpinrud.

Changes from the old system: Inherit from Non Linear Pid Controller.lvclass as the controller algorithm is the same.

This is a type of optimal control. The controller equation is based on the non-linear PID controller. The main difference between LQR and its parent Non Linear PID Controller is the automatic tuning of the matrices K_P and K_D.

Constructor Summary

| Name | Note |
| --- | --- |
| LQR() | Default constructor to create an instance of the class. |
| LQR (Matrix K_i, Matrix Q, Matrix R) | Constructor which gets and sets the tuning matrices. |

Field Summary

| Data Type | Name | Note |
| --- | --- | --- |
| Matrix | Q | Tuning matrix for Riccati equation |
| Matrix | R | Tuning matrix for Riccati equation |

Method Summary

| Data Type of Return | Name with input arguments | Note |
| --- | --- | --- |
| Vessel.lvclass | Calculate Forces (Vessel.lvclass vessel) | Calculates the desired forces for the vessel. This method also calls on the parent calculate Forces () in order to find the forces. |
| Void | Set Gains LQR (Matrix K_I Matrix Q, Matrix R, Matrix K_anti, 1D Array of Double Integral Limit) | Sets the tuning matrices for the controller. |

Sliding Mode.lvclass

Labview Object.lvclass
	∟ Controller.lvclass
		∟ Sliding Mode.lvclass

Implemented by Viktor Berg.

Constructor Summary

| Name | Note |
| --- | --- |
| Sliding Mode () | Default constructor to create an instance of the class. |
| Sliding Mode (Matrix K_s, Matrix K_pid, 1D Array of Double Lambda, 1D Array of Double Phi) | Constructor that gets and sets the tuning matrices. |

Field Summary

| Data Type | Name | Note |
| --- | --- | --- |
| 1D Array of Double | Int_limit | Used to check if the integral term shall integrate the error. If the error is larger than the Integral limit, zero is sent to the integrator. |
| Matrix | K_pid | Tuning matrix for the standard PID controller used in the algorithm. |
| Matrix | K_s | Tuning Matrix |
| 1D Array of Double | Lambda | Tuning Matrix |
| 1D Array of Double | Phi | Tuning Matrix |

Method Summary

| Data Type of Return | Name | Note |
| --- | --- | --- |
| Vessel.lvclass | Calculate Desired States (Vessel.lvclass vessel) | Calculates the desired position and velocity for the vessel. This method must be implemented by all children of this class. |
| Void | Set Gains (Matrix K_s, Matrix K_pid, 1D Array of Double Lambda, 1D Array of Double Phi, 1D Array of Double Integral Limit) | Sets the tuning matrices from the corresponding input. |

Backstepping.lvclass

Labview Object.lvclass
	∟ Controller.lvclass
		∟ Backstepping.lvclass

Not implemented yet!

Constructor Summary

| Name | Note |
| --- | --- |
| Backstepping() | Default constructor to create an instance of the class. |

Method Summary

| Data Type of Return | Name with input arguments | Note |
| --- | --- | --- |
| Vessel.lvclass | Calculate Desired States (Vessel.lvclass vessel) | Calculates the desired forces for the vessel. |

Adaptive Backstepping.lvclass

Labview Object.lvclass
	∟ Controller.lvclass
		∟ Backstepping.lvclass
			∟ Adaptive Backstepping.lvclass

Not implemented yet!

Stand-Alone Methods

mainVI.vi

This is a stand-alone VI used when running the system together with a compactRIO unit. The VI starts by reading a configuration file and setting up TCP/IP connections to the user interface. Four threads is then started to run concurrently sharing the same supervisor object. Mutual Exclusion mechanisms are implemented to avoid race conditions. The locks have time out limits in order to avoid dead locks. Starvation can occur, with wrong timing.

mainVI_SIL.vi

This is a stripped down version of the mainVI.vi as it is to be run with the simulator “in place”. Three of the four threads are moved out to the simulator environment to ensure concurrent running. Only the “main loop” remains.

Reference Model

This is a polymorphic VI which contains a library of reference models used in different settings. The polymorphic VI can change between different VIs depending on the input, but can also be set to a fixed through a selector in the block diagram.

Method Summary

| Data Type of Return | Name | Note |
| --- | --- | --- |
|  | Reference Model Altitude | Filter based method, scalar (Not part of polymorphic VI) |
|  | Reference Model DP | Filter based method, vector |
|  | Reference Model Joystick | Filter based method, vector |
|  | Reference Model Tracking | Filter based method, vector |

Kinematics

Rotation matrix J(Θ)=R(Θ)03×303×3T(Θ)

| Data Type of Return | Name | Note |
| --- | --- | --- |
| Matrix | RotationMatrix (1D Array of Double eta) | A polymorphic VI for Rotation matrices. A selector specifies which of the two RotationMatrix~ methods to be run. |
| Matrix | RotationMatrix6DOF (1D Array of Double eta) | Gives the rotation matrix in Euler angles for a 6 DOF system based on the position vector eta given in Euler angles. |
| Matrix | RotationMatrixQuaternions (1D Array of Double eta) | Gives the rotation matrix in quaternions for a 6DOF system based on the position vector eta given in quaternions. |
| Matrix | TransformationMatrix (1D Array of Double eta) | A polymorphic VI for Transformation matrices. A selector specifies which of the two TransformationMatrix~ methods to be run. |
| Matrix | TransformationMatrixEuler (1D Array of Double eta) | Gives the transformation matrix in Euler angles for a 6 DOF system based on the position vector eta given in Euler angels. |
| Matrix | TransformationMatrix Quaternions (1D Array of Double eta) | Gives the transformation matrix in quaternions for a 6 DOF system based on the position vector eta given in quaternions. |
| Matrix | SkewMatrix (1D Array of Double) | A polymorphic VI for creating a Skew matrix based on the input vector. A selector determines which of the 3DOF or 6DOF version to run. |
| Matrix | SkewMatrix3DOF (1D Array of Double vector) | The input vector must be of size 3. These elements are then used to create the Skew Matrix. |
| Matrix | SkewMatrix6DOF (1D Array of Double nu) | The input vector should be of size 6. The three last elements in the input vector are used to set up the skew matrix. |
| Matrix | J Library (1D Array of Double eta) | A polymorphic VI for creating the rotation matrix J. A Selector decides what form of the J matrix to be sent as an output |
| Matrix | J_euler (1D Array of Double eta) | Creates the rotation matrix J for 6DOF system using Euler angles. |
| Matrix | J_quart (1D Array of Double eta) | Creates the rotation matrix J for 6DOF system using quaternions. |
| Matrix | Jtrans_euler (1D Array of Double eta) | Creates the rotation matrix J for 6DOF system using Euler angels. Returns the transposed of J. |
| Matrix | Jtrans_quart (1D Array of Double eta) | Creates the rotation matrix J for 6 DOF system using quaternions. Returns the transposed of J. |
| Matrix | Jinv_euler (1D Array of Double eta) | Creates the rotation matrix J for 6DOF system using Euler angles. Returns the inverse of J. |
| Matrix | Jinv_euler (1D Array of Double eta) | Creates the rotation matrix J for 6 DOF system using quaternions. Returns the inverse of J. |
| Matrix | Jdot (1D Array of Double eta, 1D Array of Double nu) | Creates the time derived of rotation matrix by calculating R and T. Only Euler angles are used for this. |
| 1D Array of Double | EtaTransformation (1D Array of Double eta) | Transform a vector between Euler angles and quaternions using either Eta_Euler2QEta_Quart or Eta_Quart2Eta_Euler depending on transformation direction. |
| 1D Array of Double | Eta_Euler2Eta_Quart (1D Array of Double eta) | Transforms the input vector given in NED + Euler angels to a vector given with NED + quaternions. |
| 1D Array of Double | Eta_Quart2Eta_Euler (1D Array of Double eta) | Transforms the input vector given with NED + quaternions to a vector in NED + Euler angles. |
| 1D Array of Double | euler2q (1D Array of Double eta) | Calculates the angles in quaternions from Euler angles. Method is taken from MSS toolbox written by Thor I. Fossen. |
| 1D Array of Double | q2Euler(1D Array of Double eta) | Calculates the angles in Euler angels from quaternions. The method is taken from MSS toolbox written by Thor I. Fosse and translated to LabView code. |

Other

| Data Type of Return | Name | Note |
| --- | --- | --- |
| - | Rad 2 PI-PI | Polymorphic VI for the two following methods. |
| Double | Rad 2 PI-PI Scalar (Double) | Converts the angle in radians to the interval –π to π. |
| 1D Array of Double | Rad 2 PI-PI Vector(1D Array of Double) | Converts angles in radians to the interval –π to π. |

Integrators

| Data Type of Return | Name | Note |
| --- | --- | --- |
| Double/1D Array of Double | Integrator Trapezoid | A Polymorphic VI for the trapezoid integrators in the system. Both the input and a selector can be used to determine the method to be used. |
| Double | Integrator Trapezoid scalar | A Scalar integrator using the trapezoidal rule. |
| 1D Array of Double | Integrator Trapezoid vector | A vector integrator using the trapezoidal rule. |

Signal Processing

| Data Type of Return | Name | Note |
| --- | --- | --- |
|  | Alt_input |  |
|  | Alt_IO |  |
|  | Cocoord |  |
|  | Configure sub-VI |  |
|  | CRIO write |  |
|  | DVL_2_QUAD |  |
|  | Input converter |  |
|  | JS chair get |  |
|  | MRU_IO |  |
|  | MRU_SP |  |
|  | MRUinput subVI |  |
|  | MT_DATA_TRANS |  |
|  | MT_GET |  |
|  | MT MOD |  |
|  | NAVI_IO 2 |  |
|  | NAVIpacinput subVI |  |
|  | NAVIpacinput subVI 30k |  |
|  | Origin SP |  |
|  | Port 3 IO |  |
|  | ROV_IO 2 |  |
|  | ROV_IO 30k |  |
|  | ROVinput 30k subVI |  |
|  | ROVinput subVI |  |
|  | Search string |  |
|  | Set Length |  |
|  | Signal IO |  |
|  | Signal IO SIL |  |
|  | SP BC |  |
|  | SP_mod |  |
|  | Write sub-VI |  |

Others

| Data Type of Return | Name | Note |
| --- | --- | --- |
| Double Matrix | AutoParameter Guidance Omega (1D Array of Double eta_ref, 1D Array of Double eta_last) | Calculates desired omega values for the filter reference model. |
| 1D Array of Double | Error Calculations (1D Array of Double eta, 1D Array of Double eta_des) | Used to calculate the difference between the current position and the desired position. |
| 1D Array of Double | Eta2cont | Transform a discrete angle to continuous angle. Should not be placed such that it can miss any time step! |
| Double, Boolean | Get_set_point(Double setpoint, Boolean flag) | When flag is true, the current setpoint is stored and sent as output until flag is true again. (Switch on, switch off mechanism) |
| Matrix, Matrix | LQR | Used to calculate the tuning matrices for the LQR controller. (May move to class) |
| Matrix | Mass 2 Coriolis | Used to calculate the Coriolis matrix. Used in Non Linear PID and Non Linear Passive Observer. |
| String | Read Node Name (Tag Reference ref, String AttributeName) | Reads the attribute matching the field AttributeName in the tag ref points to. Used in the configuration file reader. |
| TCP Connection Reference | Reconnection Procedure | Polymorphic VI for the two following methods. |
| TCP Connection Reference | Reconnection Procedure Scalar (TCP Connection Reference ref, UInt16 Port) | Checks if connection is lost, if so tries to reconnect to client. |
| 1D Array of TCP Connection Reference | Reconnection Procedure Array (1D Array of TCP Connection Reference ref, 1D Array of UInt16 ports) | Checks if the connection is lost, if so tries to reconnect to client. All connections must be reestablished or none are. |
| String | Scan Token Generator | Generates the scan-string used to read integer and double values from a string. Used in the String to Matrix methods. |
| 1D Array of UInt16 | Sperre thrust alloc Minerva | Converts the desired actions from the control system to a telebuffer/telegram which is to be sent to the vessel. |
| 1D Array of UInt16 | Sperre thrust alloc Neptune | Converts the desired actions from the control system to a telebuffer/telegram which is to be sent to the vessel. |
| Matrix | String to Matrix | A polymorphic VI that transform a String of numbers to a Matrix. |
| Matrix | String to Matrix Diag | Transforms a String of number to a diagonal matrix where the numbers in the String is the values on the diagonal in the matrix. |
| Matrix | String to Matrix Norm | Transforms a String of number to a matrix where each row in the matrix is separated by “;” in the String. |
| 1D Array of Double | String to Vector | Transforms a String of numbers to a 1D Array of Double. |
| Boolean | ToggleSwitch | Used to convert a constant True/False Signal to a pulse True signal. |
| Boolean | True Once | Used to convert a constant True/False Signal to one pulse of True. True only once! |

Configuration File

The configuration file is unique for each vessel to be used with the control system. This is what decides the setup for the system. It is therefore important that the configuration file is correct. The format on the file is set to xml as it is easy for both machines and humans to read. Keep in mind that the tags must be exact as well as the attributes.

Table of tags and attributes

| Tag | Parent Tag(s) | Attributes | Notes |
| --- | --- | --- | --- |
| <Configuration> | - | - | Start tag for configuration |
| <Sensor> | <Configuration> | ID, Name, Type, PosX, PosY, PosZ, RotX, RotY, RotZ | Tag for initializing sensors |
| <Vessel> | <Configuration> | Name, Type, NDOF | Tag for vessel initialization. |
| <TechincalData> | <Vessel> | DOF, Length, Width, Height, Buoyancy, CenterGravity, CenterBuoyancy, TimeStep, TauMax | Tag used for vessel data |
| <PortConfiguration> | <Vessel> | - | Tag for initialization of communication ports for compactRIO unit |
| <Port> | <PortConfiguration> | ID, BaudRate, DataBits, Parity, StopBits, FlowControl | Tag for data used to set up a port for the compactRIO |
| <Matrix> | <Vessel> | - | Tag for matrix |
| <Mass> | <Matrix> | Type, Size, Values | Tag for Mass matrix |
| <AddedMass> | <Matrix> | Type, Size, Values | Tag for Added Mass matrix |
| <LinearDamping> | <Matrix> | Type, Size, Values | Tag for Linear Damping matrix |
| <NonLinearDamping> | <Matrix> | Type, Size, Values | Tag for Non Linear Damping matrix |
| <Observer> | <Vessel> | - | Tag for observer collection |
| <KalmanFilter> | <Observer> | Mode | Tag for Kalman Filter Observer |
| <KalmanFilterAltitude> | <Observer> | Mode | Tag for Altitude Kalman Filter Observer |
| <Passive> | <Observer> | Mode | Tag for Passive Non Linear Observer |
| <TuningKalman> | <KalmanFilter>, <KalmanFilterAltitude> | - | Tag for tuning fields for Kalman Filters |
| <TuningPassive> | <Passive> | - | Tag for tuning fields for Kalman Filters. |
| <R> | <TuningKalman> | Type, Size, Value | Tag for tuning matrix R |
| <Q> | <TuningKalman> | Type, Size, Value | Tag for tuning matrix Q |
| <Wave> | <TuningPassive> | TypeOmega, SizeOmega, ValueOmega, TypeDelta, SizeDelta, ValueDelta, TypeLambda, SizeLambda, ValueLambda | Tag for wave filter values |
| <K1> | <TuningPassive> | Type1, Size1, Value1, Type2, Size2, Value2 | Tag for tuning matrix K1. Consists of two 6x6 matrices. Can be set to zero as they will be calculated in the control system by tuning equations. |
| <K2> | <TuningPassive> | Type, Size, Value | Tag for tuning matrix K2. |
| <K3> | <TuningPassive> | Type, Size, Value | Tag for tuning matrix K3 |
| <K4> | <TuningPassive> | Type, Size, Value | Tag for tuning matrix K4 |
| <T> | <TuningPassive> | Type, Size, Value | Tag for time constant matrix T |
| <Navigation> | <Vessel> | - | Tag for navigation collection |
| <DPOperation> | <Navigation> | Mode | Tag for DP Guidance |
| <Tracking> | <Navigation> | Mode | Tag for Tracking Guidance |
| <TrackingSTRM> | <Navigation> | Mode | Tag for Tracking Guidance with synthetic reference model |
| <Joystick> | <Navigation> | Mode | Tag for Joystick Guidance |
| <Criteria> | <DPOperation>, <Tracking>, <Joystick> | SizeRestart, Restart, SizeWait, Wait | Tag for restart and wait criteria values for guidance objects. |
| <Omega> | <DPOperation>, <Tracking>, <Joystick> | Type, Size, Value | Tag for reference model tuning matrix omega. |
| <Delta> | <DPOperation>, <Tracking>, <Joystick> | Type, Size, Value | Tag for reference model damping matrix |
| <Controller> | <Vessel> | - | Tag for controller collection |
| <PIDLinear> | <Controller> | Mode | Tag for Linear PID Controller |
| <NonLinearPID> | <Controller> | Mode | Tag for Non Linear PID Controller |
| <LQR> | <Controller> | Mode | Tag for LQR Controller |
| <SlidingMode> | <Controller> | Mode | Tag for Sliding Mode Controller |
| <TuningPID> | <PIDLinear>, <PIDNonLinear>, <LQR> | - | Tag for tuning fields for PID based controllers. These include linear, non-linear PID as well as LQR |
| <TuningSliding> | <SlidingMode> | - | Tag for tuning fields for Sliding Mode controller. |
| <KP> | <TuningPID> | Type, Size, Value | Tag for Proportional term tuning matrix |
| <KI> | <TuningPID> | Type, Size, Value | Tag for Integral term tuning matrix |
| <KD> | <TuningPID> | Type, Size, Value | Tag for Derivative term tuning matrix |
| <R> | <TuningPID> | Type, Size, Value | Tag for tuning matrix R used in LQR computations. |
| <Q> | <TuningPID> | Type, Size, Value | Tag for tuning matrix Q used in LQR computations. |
| <KS> | <TuningSliding> | Type, Size, Value | Tag for tuning matrix in Sliding Mode. |
| <KPID> | <TuningSliding> | Type, Size, Value | Tag for PID tuning matrix used in Sliding Mode controller. |
| <Lambda> | <TuningSliding> | Value | Tag for tuning in Sliding Mode |
| <Phi> | <TuningSliding> | Value | Tag for tuning in Sliding Mode |
| <IntegralLimit> | <TuningSliding> | Value | Tag for integral limits set on the integrator |
| <SensorInstalled> | <Vessel> | NumberOfSensors, SensorIDs | Tag used to set which sensors are used by the vessel. |
| <Thruster> | <Vessel> | ID, Name, Type, PosX, PosY, PosZ, RotTheta, RotPhi, MaxRPM, Coefficient | Tag for Thruster data. |